6.2 Menu & Toolbar

The following explains about menus and toolbar

6.2.1 Menus

Name and function of menus are as follows.

| Menu | Sub-Menu | Function |
| --- | --- | --- |
| File | New Project | Creates a new project |
|  | Open Project | Opens an existing project. |
|  | Save Project | Saves current project |
|  | Save As | Saves current project with a new name |
|  | Option | Specifies Auto-saving time interval, background color of each window, tag's color, and so on |
|  | Change PLC Type | Changes the kind of PLC to connect |
|  | Change COM Port | Changes the COM Port of PC |
|  | Print | Prints a selected screen |
|  | Print Preview | Previews the screen to be printed |
|  | Print setup | Specifies print options |
|  | Exit | Quit Panel Editor |
| Edit | Cut | Cuts a selected tag and places it on the clipboard. |
|  | Copy | Copies a selected tag and place it on the clipboard. |
|  | Paste | Pastes a tag from clipboard to screen |
|  | Delete | Deletes a selected tag |
|  | Layer | Allows user to arrange tags on different layers.. |
|  | Align/Space | Aligns multiple tags by same reference point. |
|  | Grid and Snap | Displays or hides grid, sets an interval of grid and makes the starting point of the tag come on a grids when 'snap to grid' is checked |
|  | Select All | Selects all tags on the screen |
|  | Lock Tag | Locks properties and position of selected tags |
|  | Unlock Tag | Unlocks every fixed tags on current screen |
| View | Toolbar | Displays or hides toolbar in menu window. |
|  | Status Bar | Displays or hides Status bar |
|  | Memory Usage | Displays current memory usage |

| Menu | Sub-Menu | Function |
| --- | --- | --- |
| Screen | New Screen | Creates a new screen |
|  | Delete Screen | Deletes selected screen |
| Tag | Selection | Changes to an arrow mode |
|  | Digit | Places digit tag on screen |
|  | Text | Places text tag on screen |
|  | Massage | Places massage tag on screen. |
|  | Button | Places button tag on screen |
|  | Lamp | Places lamp tag on screen |
|  | Bar Graph | Places bar graph tag on screen |
|  | Trend Graph | Places trend graph tag on screen |
|  | Pie Graph | Places pie graph tag on screen |
|  | Rotate | Places rotating tag on screen |
|  | Move | Places moving tag on screen |
|  | Clock | Places clock tag on screen |
|  | Image | Places bitmap image on screen |
|  | Line | Draws a line onto screen |
|  | Rect. | Draws a rectangle onto screen |
|  | Circle | Draws circle onto screen |
| Common Resource | Massage | Manages the massage list |
|  | Image | Manages bitmap image files |
|  | Parameter | Specifies parameters |
|  | Alarm | Specifies alarm function |
|  | Calculation | Specifies calculation function |
|  | Block communication | Specifies block communication function |
|  | Reservation | Specifies reservation function.(B Type Only) |
| Communi- cation | Download | Downloads current project at XGT-Panel |
|  | Upload | Uploads project from XGT-Panel. |
|  | Font | Downloads selected font at XGT-Panel |
|  | Date/Time | Sets time/date of XGT-Panel.(B Type only) |

| Menu | Sub-Menu | Function |
| --- | --- | --- |
| Communi-cation | Password | Specifies password for up/download |
|  | Clear Password | Clears password for up/download |
| Help | Help Contents | Displays the Help-Contents of Panel Editor |
|  | About Panel Editor | Displays a program information, version number and copyright of Panel Editor |

6.2.2  Toolbar

Icons and functions of toolbar are as follows.

| Icon | Function | Icon | Function |
| --- | --- | --- | --- |
|  | New project |  | Selection |
|  | Open Project |  | Digit tag |
|  | Save project |  | Text tag |
|  | Cut |  | Massage tag |
|  | Copy |  | Button tag |
|  | Paste |  | Lamp tag |
|  | Print |  | Bar graph tag |
|  | New screen |  | Trend graph tag |
|  | Download |  | Pie graph tag |
|  | Grad & Snap |  | Rotate tag |
|  | Layer(Bring to front, Send to back) |  | Move tag |
|  | Align to left, right, top, bottom |  | Clock tag |
|  | Align to left, right ,top, bottom of the screen |  | Image tag |
|  | Align to vertical center, horizontal center of the screen |  | Draw Line |
|  | Evenly spaces the selected tags vertically (or horizontally) |  | Draw rectangle |
|  | Help |  | Draw circle |