Chapter 6  Panel Editor

	6.11 Key Setting


The following explains about function key.

The XGT Panel offers eight function keys as shown below..
	Key
	Description
	Initial function

	
	User defined function key 1
	None

	
	User defined function key 2
	None

	
	User defined function key 3
	None

	
	User defined function key 4
	None

	  
	Arrow key(Up)(Can be defined as a function key)
	None

	
	Arrow key(Down)(Can be defined as a function key)
	None

	
	Arrow key(Left)(Can be defined as a function key)
	Go to the previous screen

	
	Arrow key(Right)(Can be defined as a function key)
	Go to the next screen


[image: image1.png]notitie2pep - Panel Editor -[ol x|
(F) Edit(E) WView(v) Screen Tag Common Resource Communication Help(H)

AR I IR

REREERCSERERIBNEE
—

=

©

=1®I0 O O CICX -

I~ Background Image.

Sereen Praperty

Number: 0

Name: [Screend

Frevileftaron} [0 =] NewRightamow} [I =]

[Ready [D5ag8 [ Screers 1% | Suings 0% [Graphics 2%,/





A green square appears when the Mouse pointer is placed on the function key(F1).

Double click the left button of the mouse to specify function of the key. then, a following window appears.

[image: image2.png]Selectfuncti:

Bit
Diive 0ut
ncrement
Decrement
Screen

s





6.11.1 None
Specifies no function..

6.11.2 Bit
Controls a status(On or Off) of bit device.

[image: image3.png]Ackdess
s:wea functors ER—— bekdoss

i =

e

Vs gt

<ot oae

o ™ Use Password

© Tosge

& omenta On

Cancel





1) Address
Specifies a address of bit device.

Refer to Ch 6.7.5 button tag for details.

2) Value Input 

Specifies the kind of action of the function key tag when pushed..     

	Action
	Description

	On
	Turns on the bit device

	Off
	Turns off the bit device

	Toggle
	Toggles the bit device whenever pushed

	Momentary On
	Turns on the bit device while pushed.


5) Use password

If this option is specified to the function, the Write Action is disabled until password is 
unlocked
6.11.3 Drive out
Changes a current value of device to specified value.

[image: image4.png]Selectfuncti:

Diive Out -

-Value input

-Adess
Chanrel

Station

St I w

Address

150

Data

I Use Passnord

I~ Sign

& Word
© Long

Format

 DEC © HEX

Cancel





1) Address 
Specifies the address of device to drive.

Refer to Ch 6.7.2 digit tag for details.

2) Value input 
The value of specified device is changed to this value when the function key is pushed.

3) Use password 
Refer to Ch 6.11.2
4) Sign 
Enables a function to write a negative number.
Checkbox is enabled when a display format is DEC 

5) Word/Long 
Specifies data type of value
6) Format 
(1) DEC : Specifies a input value as decimal format.

(2) HEX : Specifies a input value as hexadecimal format
6.11.4 Increment
Increases the value of device as specified value whenever function key is pushed.

[image: image5.png]Selectfuncti:

-Value input

-Adess
Chanrel

Station

St I w

Address

—

Data

I Use Passnord

& Word
© Long

Format

 DEC © HEX

Cancel





1) Address 

Specifies the address of device to drive.

Refer to Ch 6.7.2 digit tag for details.

2) Value input 

The value of specified device is increased as this value when the function key is pushed.

3) Use password 

Refer to Ch 6.11.2

5) Word/Long 

Specifies data type of value

6) Format 

(1) DEC : Specifies a input value as decimal format.

(2) HEX : Specifies a input value as hexadecimal format
6.11.5 Decrement
Decreases the value of device as specified value whenever function key is pushed.

[image: image6.png]Selectfuncti:

-Value input

-Adess
Chanrel

Station

St I w

Address

—

Data

I Use Passnord

& Word
© Long

Format

 DEC © HEX

Cancel





1) Address 

Specifies the address of device to drive.

Refer to Ch 6.7.2 digit tag for details.

2) Value input 

The value of specified device is decreased as this value when the function key is pushed.

3) Use password 

Refer to Ch 6.11.2

5) Word/Long 

Specifies data type of value

6) Format 

(1) DEC : Specifies a input value as decimal format.

(2) HEX : Specifies a input value as hexadecimal format
6.11.6 Screen
Goes to the specified screen.

[image: image7.png]Lroncionkeypi]

Selectfuncti:

- Screen rumber—|

e

Data
I Use Passnord

s





1) Screen number 
Specifies the number of screen to go.

2) Use password 

Refer to Ch 6.11.2

[image: image8.bmp][image: image9.bmp]
F2





F1





F3





F4





(





(





(





(








6 -69

