# Chapter 1. General

1.1 Guide to Use This Manual

This manual includes specifications, functions and handling instructions for the XGT Panel.

This manual is divided up into chapters as follows:

| #### No. | Title | Contents |
| --- | --- | --- |
| Chapter 1 | General | Describes configuration of this manual, unit's features and terminology. |
| Chapter 2 | Names of Parts | Describes each kind of manufacturing goods, titles, and main functions |
| Chapter 3 | General Specifications | Describes general specifications of the XGT Panel. |
| Chapter 4 | System configuration | Describes system configurations in the XGT Panel. |
| Chapter 5 | Panel Editor Installation | Describes installation of Panel Editor. |
| Chapter 6 | Panel Editor | Describes function of Panel Editor. |
| Chapter 7 | XGT Panel Function | Describes function of XGT Panel. |
| Chapter 8 | Communication Specification | Describes communication Specification. |
| Appendix 1 | System Memory | Describes system memory. |
| Appendix 2 | Dimensions | Shows dimensions of the XGT Panel. |

1.2. Features

1.2.1 Features of XGT Panel

(1) Increased application design flexibility through wide graphic LCD(192× 64 dot)

(2) Slim design which considers a user's convenience

(3) Supports two communication channel independently(RS-232C, RS-422/485), ( supporting N:M communication)

(4) Built-in 512KByte Flash Memory

(5) Each memory area (Screen, Bitmap, String, Parameter, Global, Constant ) can be up/downloaded in part

(6) Provides various tags and drawing functions

(7) Supports two languages at the same time

- English is provided as a default font.

- Supports the Korean or the Chinese font additionally by downloading

(8) Provides 1000 words of system memory area.

(9) Flexibility of power supply

- You can connect the external 24VDC through a power supply terminal.

- The 5VDC power can be supplied directly through a communication port(RS-232C)(Available on LG PLC and LG INV )

(10) Supports very high communication speed (115,200 bps max. )

(11) Provides user defined function key for each screen : F1~F4, , , , 

(12) Provides time, date, clock function (Available on XPE-BK1B only)

(13) Supports a two levels of password (The password which permits project access/tag access )

(14) Supports user defined bitmap.

(15) Provides various communication driver.