6.7 Tag menu

The following explains about various tags.

XGT-Panel supports 15 kinds of tags. There are some differences in properties of tags according to PLC type to connect, and the example of this manual is the case of MASTER-K series

6.7.1 Selection

Selects tags on screen to edit.

6.7.2 Digit tag (  )

Displays a device value of PLC or writes value to device

< Properties of Digit tag >

1) Location

Indicates tag’s location.(X-Axis:0~191, Y-Axis:0~63)

Relocating by mouse drag is available.

2) Effect

(1) small font : Changes the size of character to 6 x 8 dot. Default size is 8 x 16 dot.

(2) Double : Changes the size of character to 16 x 32 dot.

(3) Reverse : Reverses the tag's color.

3) Address

(1) Channel : Specifies communication channel of digit tag

① S Area

• specifies to an system memory of XGT Panel.

• An system memory is the memory to be provided to the user, and allows user to use as user memory or as system flags

• The size of system memory is 1,000 Word(0~999), and you can use 900 Word(0~899) as user memory area.

the remains are system flag area. A latch area can be set in the case of B type.

• Refer to 6.8.3 Parameter for details. And refer to appendix for details about system flags.

• Click , then The following window appears and can input an address or system flag.

② Ch 1

• Specify the channel as Ch 1(RS-232C).

• Click , then The following window appears and can input an address to read.

③ Ch 2

• Specify the channel as Ch 2(RS-422/485).

• Click , then The following window appears and can input an address to read.

(2) Station : Specifies a station number to connect.

•.enabled when channel is specified as Ch.1 or Ch.2

4) Data

(1) Write enable : Enables Write action of tag by key input

•. If the SET key of XGT Panel is pushed, A cursor appears on the tag which [Write Enable] is specified.

•. The cursor moves in order from a left upside to right downside direction whenever SET KEY is pushed.

•. Change the value of the tag which is focused currently by direction keys(▲,▼,◀,▶).

•. When the ENT Key is pressed, XGT-Panel writes the inputted value to PLC.

•. Does not operate if the inputted value is not correct

- (2) Sign : Enables tag to display a negative number.

•. Checkbox is enabled when a display format is DEC

(3) Use password : If this option is specified to tag, the Write Action is disabled until password is unlocked.

•. Checkbox is enabled when [Write Enable] is permitted

(4) Max. : Specifies the maximum value which is available to input.

•. Editbox is enabled when [Write Enable] is permitted

•. A larger value than specified Max.Value can not be writed to PLC. Refer to following table for details.

(5) Min. : Specifies the minimum value which is available to input.

•. Editbox is enabled when [Write Enable] is permitted

•. A smaller value than specified Min.Value can not be writed to PLC. Refer to following table for details.

(6) Word/Long : Specifies data type of digit tag

•. Word : 2 Bytes, Long : 4 Bytes

| Data |  | Range |  |
| --- | --- | --- | --- |
| Display Format | Type |  |  |
| DEC | Word | Signed | -32768 ~ 32767 |
|  |  | Unsigned | 0 ~ 65535 |
|  | Long | Signed | -2147483648 ~ 2147483647 |
|  |  | Unsigned | 0 ~ 4294967295 |
| HEX | Word | 0 ~ FFFF |  |
|  | Long | 0 ~ FFFFFFFF |  |
| BCD | Word | 0 ~ 9999 |  |
|  | Long | 0 ~ 99999999 |  |

< Data display range >

5) Display format

(1) DEC : Displays by decimal format.

(2) HEX : Displays by hexadecimal format

(3) BCD : Displays by binary coded decimal format.

(4) Total digit : Specifies the number of digits allowed to be displayed or entered.

(Ex) Actual value of device : 12345 , Total digit : 3 → Displayed value : 345.

(5) Fraction digit : Specifies the number of digits to the right of decimal point allowed to be displayed or entered.

• The combobox is enabled when the display mode is DEC

|  |  | Available maximum number of digit |  | Display example |
| --- | --- | --- | --- | --- |
|  |  | Total | Fractional |  |
| Word | DEC | 5 | 4 | 0.1234 |
|  | HEX | 4 | Not available | FFFF |
|  | BCD | 4 | Not available | 0123 |
| Long | DEC | 10 | 9 | 0.123456789 |
|  | HEX | 8 | Not available | FFFFFFFF |
|  | BCD | 8 | Not available | 10234567 |

< Example of display format >

(6) Outline : The digit tag is outlined with the solid-line.

(7) Fill leading zeroes : Displays leading zeroes.

(Ex) The case of 123 → Displays as 00123(When the total digit is specified as 5.)

6) Example of the Writing Action.

(1) System configuration

MK120S(St.1)

RS-485

XGT Panel(St.0)

(2) Executes Panel Editor and specify the PLC Type of Ch 2 as LG:MASTER-K(Link).

• Make the communication parameter of GLOFA-Panel and Master K120S same.

• Refer to Master K120S user manual for communication setting of Master-K120S.

(3) Property of Digit tag

• Channel : Ch 2(RS-422/485)

• Station : 1

• Address : D0000

• Display format : Word, Dec

• Max. : 65535

• Min. : 0

(4) Downloading Project

• Download the created project at XGT-Panel.

• Refer to Ch. 6.9 for details about download.

(5) Writing the value to the PLC

Assume that a current value of tag is 12345.

12345

| Key | Display |  |  |  |  | Description |
| --- | --- | --- | --- | --- | --- | --- |
| 『SET』 |  |  |  |  |  | A cursor appears on the tag when the SET key of GLOFA-Panel is pushed. |
|  | 1 | 2 | 3 | 4 | 5 |  |
| 『』 |  |  |  |  |  | The cursor moves to a ten's place. (The cursor flickers) |
|  | 1 | 2 | 3 | 4 | 5 |  |
| 『』 |  |  |  |  |  | The cursor moves to a one's place. (The cursor flickers) |
|  | 1 | 2 | 3 | 4 | 5 |  |
| 『』 |  |  |  |  |  | The value to write increases as 1. |
|  | 1 | 2 | 3 | 4 | 6 |  |
| 『』 |  |  |  |  |  | The value to write decreases as 1. |
|  | 1 | 2 | 3 | 4 | 5 |  |
| 『』 |  |  |  |  |  | The value to write increases as 1. |
|  | 1 | 2 | 3 | 4 | 6 |  |
| 『ENT』 |  |  |  |  |  | Writes the value to PLC |
|  | 1 | 2 | 3 | 4 | 6 |  |

The device value of D0000 of MK-120S is changed to 12346.

6.7.3 Text tag (  )

Displays text

< Property of text tag >

1) Location

Indicates tag’s location (X-Axis:0~191, Y-Axis:0~63)

Relocating by mouse drag is available.

2) Effect

(1) small font : Changes the size of character to 6 x 8 dot. Default size is 8 x 16 dot.

(2) Double : Changes the size of character to 16 x 32 dot.

(3) Reverse : Reverses the tag's color.

3) Text

Input the text to display.(Can display up to 24 letters.)

4) Outline

The text tag is outlined with the solid-line

6.7.4 Message tag (  )

• Displays a registered message according to value of device.

• Displays a blank if a registered message for the current value of device does not exist

< Property of message tag >

1) Location

Indicates tag’s location (X-Axis:0~191, Y-Axis:0~63)

Relocating by mouse drag is available.

2) Effect

(1) small font : Changes the size of character to 6 x 8 dot. Default size is 8 x 16 dot.

(2) Double : Changes the size of character to 16 x 32 dot.

(3) Reverse : Reverses the tag's color.

3) Address

Refer to Ch 6.7.2 Digit tag for details.

4) Edit message

Edits the content of a message tag. Click the [Edit Message] button to edit contents of message.

• Message List(Right grid) : Shows content and ID of all messages.

- Click a message button, then you can modify this content.

- Refer to Ch. 6.8.1 Message Management for details.

• Registered Message(Left grid) : Shows the messages which are registered for the selected message tag.

- Up to fifty messages can be registered to one message tag. All messages have to contain ID and device value

• Add : Registers the message of message list at a selected message tag.

• Sort : Aligns registered messages by [Value].

• Message : Displays a management screen of messages.

- Refer to Ch. 6.8.1 Message Management for details

• Cancel : Cancels a message editing.

• OK : Completes a message editing..

6.7.5 Button tag (  )

Displays and controls a status(On or Off) of bit device

< Property of a button tag >

1) Location

Indicates tag’s location (X-Axis:0~191, Y-Axis:0~63)

Relocating by mouse drag is available.

2) Effect

(1) Double : Changes the size of character to double.

3) Address

(1) Channel : Specifies communication channel of digit tag

① S Area

• specifies to an system memory of XGT Panel.

• An system memory is the memory to be provided to the user, and allows user to use as user memory or as system flags

• The size of system memory is 1,000 Word(0~999), and you can use 900 Word(0~899) as user memory area.

• The remains are system flag area. A latch area can be set in the case of B type.

• Refer to 6.8.3 Parameter for details and refer to appendix for details about system flags.

• Click , then The following window appears and can input an address or system flag.

• Add the bit position at the back of the word address to specify a specific bit of an system memory.

(Example)  8th bit of 120th word : 1208

12th bit of 700th word: 700C

② Ch 1

• Specify the channel as Ch 1(RS-232C).

• Click , then The following window appears and can input an address to read.

③ Ch 2

• Specify the channel as Ch 2(RS-422/485).

• Click , then The following window appears and can input an address to read.

(2) Station : Specifies a station number to connect.

•.enabled when channel is specified as Ch.1 or Ch.2

4) Action

Specifies the kind of action of button tag when pushed..

| Action | Description | Remark |
| --- | --- | --- |
| On | Turns on the bit device | 1) Choose tag by "SET" key. 2) Operate by "ENT" key. |
| Off | Turns off the bit device |  |
| Toggle | Toggles the bit device whenever pushed |  |
| Momentary On | Turns on the bit device while pushed. |  |

5) Use password

If this option is specified to tag, the Write Action is disabled until password is unlocked.

6) On Value & Shape

Specifies display format of device status.

| On Value | Actual value of device |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 |  | 0 |  |
|  | Circle | Rectangle | Circle | Rectangle |
| “0” |  |  |  |  |
| “1” |  |  |  |  |

6.7.6 Lamp tag (  )

Displays a status(On or Off) of bit device.

< Property of a lamp tag >

1) Location

Indicates tag’s location (X-Axis:0~191, Y-Axis:0~63)

Relocating by mouse drag is available.

2) Effect

(1) Double : Changes the size of character to double.

3) Address

Refer to Ch.6.7.5 Button tag.

4) On Value & Shape

Specifies display format of device status.

| On Value | Actual value of device |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 |  | 0 |  |
|  | Circle | Rectangle | Circle | Rectangle |
| “0” |  |  |  |  |
| “1” |  |  |  |  |

6.7.7 Bar graph tag (  )

Displays current value of device as a shape of bar-graph

< Property of bar graph tag >

1) Location

• Indicates tag’s location (X-Axis:0~191, Y-Axis:0~63)

• Relocating by mouse drag is available.

2) Address

Refer to Ch 6.7.2 Digit tag for details

3) Data

(1) Sign : Enables tag to display a negative number

(2) Max : Specifies a value which a graph becomes the maximum.

(3) Min : Specifies a value which a graph becomes the minimum.

• When the value of device is out of the specified Min/Max range, Bar-graph tag displays Min/Max value for each

• A minimum value can not be specified as greater value than a maximum value

(4) Word/Long : Specifies data type of digit tag(Word : 2 Bytes, Long : 4 Bytes)

| Data | Min/Max value |  |
| --- | --- | --- |
|  | Signed | Unsigned |
| Word | -32768 ~ 32767 | 0 ~ 65535 |
| Long | -2147483648 ~ 2147483647 | 0 ~ 4294967295 |

4) Display format

(1) Width & Height

• Specifies width and height of bar graph

• Width : 8~192 , Height : 8~64.

• Size adjustment by mouse drag is available.

(2) Direction

• Specifies a progress direction of the graph according to the increase of value of device.

<Display format of bar graph tag>

5) Example of bar graph tag

• Assume that properties of bar graph are specified as shown in the below.

• A Bar Graph Tag operates as shown below according to the value of D0000 of the station 1.

| D0000 = 0 | D0000 = 50 | D0000 = 75 | D0000 >= 100 |
| --- | --- | --- | --- |

6.7.8 Trend graph tag (  )

Displays current value of device as a shape of trend graph.

< Property of trend graph tag >

1) Location

• Indicates tag’s location (X-Axis:0~191, Y-Axis:0~63)

• Relocating by mouse drag is available.

2) Address

Refer to Ch 6.7.2 Digit tag for details

3) Data

Refer to Ch 6.7.7 Digit tag for details.

4) Display format

(1) Width & Height

• Specifies width and height of bar graph

• Width : 8~192 , Height : 8~64.

• Size adjustment by mouse drag is available.

5) Sampling number

Specifies a maximum number of the section which the X-axis of trend curve has.

6) Sampling time

Specifies a displaying time interval (1~3600 seconds)

7) Examples

Assume that the properties of bar graph are specified as shown below and the value of D0000 of Station 0 increases ten

per every five seconds.

0

5

10

15

20

25

30

35

40

45

50

Time(Sec)

Value

50

40

30

20

10

• The total time to display = (Sampling time) x (Sampling number) = 5 x 10 = 50seconds

6.7.9 Pie graph tag (  )

• Displays current value of device as a shape of pie graph.

• Various starting angle and displaying angle size can be specified as shown above.

< Property of pie graph tag >

1) Location

• Indicates tag’s location (X-Axis:0~191, Y-Axis:0~63)

• Relocating by mouse drag is available.

2) Effect

(1) Reverse : Reverses the tag's color.

3) Address

Refer to Ch 6.7.2 Digit tag for details.

4) Data

Refer to Ch 6.7.7 Bar Graph Tag for details.

5) Starting angle & Max. Angle

| Angle(degree) | Starting Angle(Max. Angle : 180°) | Maximum Angle(Starting Angle : 0°) |
| --- | --- | --- |
| 0 or 360 |  |  |
| 45 |  |  |

| Angle(degree) | Starting Angle(Max. Angle : 180°) | Maximum Angle(Starting Angle : 0°) |
| --- | --- | --- |
| 90 |  |  |
| 135 |  |  |
| 180 |  |  |
| 225 |  |  |
| 270 |  |  |
| 315 |  |  |

6) Radius

• Specifies a radius of tag.

• Size adjustment by mouse drag is available

7) Pattern

| Fill | Needle |
| --- | --- |

8) Example

• Assume that properties of pie graph tag are specified as shown in the below

• A pie graph tag operates as shown below according to the value of D0000 of the station 1

| D0000 = 25 | D0000 = 50 | D0000 = 75 | D0000 >= 100 |
| --- | --- | --- | --- |

6.7.10 Rotate tag (  )

Varies a rotating speed of image of motor according to the value of device.

< Property of rotate tag >

1) Location

• Indicates tag’s location (X-Axis:0~191, Y-Axis:0~63)

• Relocating by mouse drag is available.

2) Effect

(1) Double : Changes the size of character to double.

(2) Reverse : Reverses the tag's color.

3) Address

Refer to Ch 6.7.2 Digit tag for details.

4) Data

(1) Max. RPM : Specify the maximum speed of device which makes the ratate tag revolve most fast.

(2) Sign, Word/Long : Refer to Ch 6.7.7 Bar graph tag.

5) Direction

• Specifies a direction of rotating.

• When data is specified as [Sign],the rotate tag revolves to the reverse direction if a value of device is negative number

(Example) Data : Signed, Direction : Clockwise, Current value of device : negative number

→ Tag revolves to the CCW(counter clockwise) direction.

6) Rotating speed

• The speed which the rotate tag revolves the 90degrees is divided into 20 steps according to the value of a device.

• A speed difference of an each step is 20ms.(from 20ms to 400ms)

• For example, When a maximum RPM is specified as 100.( default unit : 100/20 = 5)

| Device value | Rotating speed | The time which a rotate tag revolves once | Device value | Rotating speed | The time which a rotate tag revolves once |
| --- | --- | --- | --- | --- | --- |
| Less than 5 | 400㎳ | 400ms × 4 = 1600ms | 91 ~ 95 | 40㎳ | 40ms × 4 = 160ms |
| 6 ~ 10 | 380㎳ | 380ms × 4 = 1520ms | Greater than 96 | 20㎳ | 20ms × 4 = 80ms |
| ~ | ~ | ~ |  |  |  |

6.7.11 Move tag(  )

Varies a position of image of tag according to the values of devices

< Property of move tag >

1) Location

• Indicates tag’s location (X-Axis:0~191, Y-Axis:0~63)

• Relocating by mouse drag is available.

2) Effect

(1) Reverse : Reverses the tag's color.

3) X/Y direction

(1) Address : Refer to Ch 6.7.2 Digit tag for details.

(2) Max. Min, Sign, Word/Long : : Refer to Ch 6.7.7 Bar graph tag for details.

(3) Width : specifies the range of the X axis which a move tag can move.

• The range of the X axis has to be smaller than (192-the location of X axis)

• For example, the range of the X axis has to be specified as smaller value than 92 when the location of X axis is 100.

• Resizing by mouse drag is available.

4) Y direction

(1) Address : Refer to Ch 6.7.2 Digit tag for details.

(2) Max. Min, Sign, Word/Long : : Refer to Ch 6.7.7 Bar graph tag for details.

(3) Height : specifies the range of the Y axis which a move tag can move.

• The range of the Y axis has to be smaller than (64-the location of Y axis)

• For example, the range of the X axis has to be specified as smaller value than 54 when the location of Y axis is 10.

• Resizing by mouse drag is available.

5) X-axis/Y-axis

• Enables or disables X axis and Y axis.

• At least. one of them has to be enabled.

6) Outline

• The tag is outlined with the solid-line.

7) Example

Assume that properties of move tag are specified as shown in the below

• The unit of movement of X axis : Max value / Width = 1000 / 100 = 10

→The tag moves a 1dot per every 10 increment of device

• The unit of movement of X axis : Max value / Height = 80 / 40 = 2

→The tag moves a 1dot per every 2 increment of device

60

50

120

20

(50,40)

(50,20)

))

(100,20)

0

40

100

6.7.12 Clock tag (  )

Displays current time and date(Available on XP10BKB/DC only)

< Property of clock tag >

1) Location

• Indicates tag’s location (X-Axis:0~191, Y-Axis:0~63)

• Relocating by mouse drag is available.

2) Effect

(1) small font : Changes the size of character to 6 x 8 dot. Default size is 8 x 16 dot.

(2) Double : Changes the size of character to 16 x 32 dot.

(3) Reverse : Reverses the tag's color.

3) Display

Specifies a display format of a clock tag.

| No. | Display format | Description | Example |
| --- | --- | --- | --- |
| 1 | YYYY/mm/DD HH:MM:SS | Year/month/date hour:minute:second | 2004/06/03 10:12:24 |
| 2 | HH:MM:SS | hour:minute:second | 10:12:24 |
| 3 | YYYY/mm/DD | Year/month/date | 2004/06/03 |
| 4 | HH:MM | Hour:minute | 10:12 |
| 5 | mm/DD | Month/day | 06/03 |
| 6 | DD/mm/YYYY | Date/month/year | 03/06/2004 |
| 7 | DD/mm | Date/month | 03/06 |
| 8 | YYYY | Year | 2004 |
| 9 | mm | Month | 06 |
| 10 | DD | Date | 03 |
| 11 | HH | Hour | 10 |
| 12 | MM | Minute | 12 |
| 13 | SS | Second | 24 |
| 14 | WWW | Day of the week | Wed |

6.7.13 Image tag (  )

Displays a bitmap image.

< Property of image tag >

1) Location

• Indicates tag’s location (X-Axis:0~191, Y-Axis:0~63)

• Relocating by mouse drag is available.

2) Effect

(1) Double : Changes the size of character to double.

(2) Reverse : Reverses the tag's color

3) Image management

Refer to Ch. 6.8.2 Image Management for details

4) Example

Choose the image by a mouse. Then, the image to be selected is displayed in the screen.

6.7.14 Line tag (  )

Draws a line of various shapes..

< Property of line tag >

1) Starting point

• Specifies a X-Y coordinates of the starting point. (Max X-Axis : 192, Y-Axis : 64)

• Relocating by mouse drag is available.

2) Ending point

• Specifies a X-Y coordinates of the ending point. (Max X-Axis : 192, Y-Axis : 64)

• Relocating by mouse drag is available.

3) Line

(1) Kind : Specifies a kind of line.(One of solid line, dotted line, dashed line)

(2) Thickness : Specifies a thickness of line.(From 1 to 5 dots)

6.7.15 Rectangle tag (  )

Draws a rectangle of various shapes.

< Property of a rectangle tag >

1) Location

• Indicates tag’s location (X-Axis:0~191, Y-Axis:0~63)

• Relocating by mouse drag is available.

2) Line

(1) Kind : Specifies a kind of outline.(One of solid line, dotted line, dashed line)

(2) Thickness : Specifies a thickness of outline.(From 1 to 5 dots)

3) Fill

Fills the inside of rectangle.

4) Width, Height

Specifies width and height of rectangle.

6.7.16 Circle tag (  )

Draws a circle of various shapes.

< Property of circle tag >

1) Location

• Indicates tag’s location (X-Axis:0~191, Y-Axis:0~63)

• Relocating by mouse drag is available.

2) Line

(1) Kind : Specifies a kind of outline.(One of solid line, dotted line, dashed line)

(2) Thickness : Specifies a thickness of outline.(From 1 to 5 dots)

3) Fill

Fills the inside of circle.

4) Width, Height

Specifies width and height of rectangle.

5) Radius

Specifies a radius of circle.