6.11 Key Setting

The following explains about function key.

The XGT Panel offers eight function keys as shown below..

| Key | Description | Initial function |
| --- | --- | --- |
|  | User defined function key 1 | None |
|  | User defined function key 2 | None |
| F1 F2 F3 | User defined function key 3 | None |
| F4 | User defined function key 4 | None |
|  | Arrow key(Up)(Can be defined as a function key) | None |
|  | Arrow key(Down)(Can be defined as a function key) | None |
|  | Arrow key(Left)(Can be defined as a function key) | Go to the previous screen |
|  | Arrow key(Right)(Can be defined as a function key) | Go to the next screen |

A green square appears when the Mouse pointer is placed on the function key(F1).

Double click the left button of the mouse to specify function of the key. then, a following window appears.

6.11.1 None

Specifies no function..

6.11.2 Bit

Controls a status(On or Off) of bit device.

1) Address

Specifies a address of bit device.

Refer to Ch 6.7.5 button tag for details.

2) Value Input

Specifies the kind of action of the function key tag when pushed..

| Action | Description |
| --- | --- |
| On | Turns on the bit device |
| Off | Turns off the bit device |
| Toggle | Toggles the bit device whenever pushed |
| Momentary On | Turns on the bit device while pushed. |

5) Use password

If this option is specified to the function, the Write Action is disabled until password is

unlocked

6.11.3 Drive out

Changes a current value of device to specified value.

1) Address

Specifies the address of device to drive.

Refer to Ch 6.7.2 digit tag for details.

2) Value input

The value of specified device is changed to this value when the function key is pushed.

3) Use password

Refer to Ch 6.11.2

4) Sign

Enables a function to write a negative number.

Checkbox is enabled when a display format is DEC

5) Word/Long

Specifies data type of value

6) Format

(1) DEC : Specifies a input value as decimal format.

(2) HEX : Specifies a input value as hexadecimal format

6.11.4 Increment

Increases the value of device as specified value whenever function key is pushed.

1) Address

Specifies the address of device to drive.

Refer to Ch 6.7.2 digit tag for details.

2) Value input

The value of specified device is increased as this value when the function key is pushed.

3) Use password

Refer to Ch 6.11.2

5) Word/Long

Specifies data type of value

6) Format

(1) DEC : Specifies a input value as decimal format.

(2) HEX : Specifies a input value as hexadecimal format

6.11.5 Decrement

Decreases the value of device as specified value whenever function key is pushed.

1) Address

Specifies the address of device to drive.

Refer to Ch 6.7.2 digit tag for details.

2) Value input

The value of specified device is decreased as this value when the function key is pushed.

3) Use password

Refer to Ch 6.11.2

5) Word/Long

Specifies data type of value

6) Format

(1) DEC : Specifies a input value as decimal format.

(2) HEX : Specifies a input value as hexadecimal format

6.11.6 Screen

Goes to the specified screen.

1) Screen number

Specifies the number of screen to go.

2) Use password

Refer to Ch 6.11.2