## Chapter 1. Introduction

## Characteristics of GMWIN

GMWIN is a programming and debugging tool for the full range of GLOFA PLC.

GMWIN software has the following features.

#### Comfortable Interface

It is possible to compile and debug several programs at the same time and maximizes user's other convenience.

#### Supporting Various Languages

It supports various languages like LD, SFC, IL. So you can select and use applicable language to a system.

LD                                                            SFC                                                     IL

#### Using Variables by Symbol

You can create a program using symbols for easy understanding and memory address is also assigned automatically. It supports various data types. So you can create high-leveled programs.

#### PLC System using Project Unit

It is very easy to create and test a program because you can include several programs in one PLC system.

#### Connecting GMWIN to PLC by Easy Network

You can download and monitor programs not only in directly connected PLC but also in a local PLC connected by network.

#### Various PLC Information

It is available to monitor various PLC states and to use this function in the program mode.

#### User-Defined Libraries

Besides standard functions and function blocks, you can define a frequently used program as one of functions or function blocks for your convenience.

## 1.2 How to Connect GMWIN to PLC

In connection GMWIN to PLC, you must previously define two options – 『Method of Connection』and 『Depth of Connection』in Connect Option.

There are four kinds of methods of connection, using RS-232C, MODEM, Communication module ( GLOFA Fnet/Mnet for PC ) and Network ( Ethernet ), and three kinds of depth of connections such as Local, Remote 1 and remote 2.

When using Windows 3.x for operating system, the connections using Communication module and Network are not supported.

Select 『Connect option』in Project - Option menu.

### Method of Connection

#### Connection using RS-232C

To use RS-232C connection, connect the RS-232C cable with the serial port in PC and serial port in PLC

◆ From the Project Menu, choose Option.

◆ Choose Connect Option tab in the Option dialog box.

◆ Select RS-232C in Method of Connection.

◆ Select Communication Port.

◆ Select Depth of Connection and click OK button.

Local Connection

◆ Select Local in Depth of Connection and click OK button.

◆ Choose Connect from the Online Menu

Remote Connection (See GLOFA Fnet/Mnet user's manual for more details),

◆ Open Network Type List and select a network type.

◆ Type a station number in Station No. Box.

◆ Select slot number.

◆ Click OK button.

◆ Choose Connect from the Online Menu.

Note

The specification of RS-232C cable connection