Chapter 6. Upload

It saves a compressed project file and source file to RAM(program area) or flash memory of PLC, and uploads a program from PLC.

## Make Upload File

You can make an upload file by selecting Make Option menu.

An upload file includes project, program, function lock and user define function used in program.

- Select the Project-Option in menu. Then Option dialog box appears.

- Check the Up-load Program in Make Option dialog box, and click OK button.

- It creates the execution file for program and Upload file with selecting Compile-Make in menu

## Write to PLC(Download)

Select the Upload program when writing a program to PLC.

- Select the Online-Write in menu.

- Select the Parameter and Program, Upload Program in the Write dialog box.

According to the program size, it runs as below.

| Program Size | The place to save upload program |
| --- | --- |
| Size of(Execution Program + Upload Program) < Size of Program RAM | It writes to RAM in CPU |
| Size of Program RAM < Size of (Execution Program + Upload Program) | If device has flash memory, it writes to flash memory after query to user |

## Read from PLC(Upload)

- Select the Project-Open From PLC in menu

- If a file had the same name in Source directory, the following dialog box appears.

- Click “All” button to monitor and online-edit.

- Uploaded Project window create, then the following dialog box appears.

- Click “Yes” button to monitor and online-edit.

- You can monitor and online-edit after making project.