1-4 Link the PMU with PLC

- Open the link editor and select ‘Serial Link’

When communicate with Cnet module of K200S, 300S, 1000S (K3F-CU2A, K3F-CU4A, K4F-CUEA, K7F-CUEA) or built-in Cnet function of K200S (K3P-07AS, K3P-07BS, K3P-07CS), choose the PLC typa as ‘K200S,K300S,K1000S(LINK)’.

In this example, assume that PMU communicate with built-in Cnet of K200S (pin number 4, 5, 7). Therefore, select the ‘K200S,K300S,K1000S(LINK)’ in the PLC type selection window.

# Note

※ When the PMU communicate with loader port of K200S CPU(2, 3, 5 pins), select the PLC type as ‘New Master-K(X)S Loader’ in link editor.

- Link each buffers with PLC address as following;

Buffer 40 ~ 1023 can be linked with PLC devices. The buffer 0 ~ 39 is system area, and it is not available for user data.

Select buffer 40 and input M001 to link the buffer 40 and M001 of PLC.

Link buffer 41 and M002 as the same way.

After serial LINK setup is completed. the buffer of PMU and PLC memory is matched as following;

| Buffer(Word) No. | Bit Number |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PMU : 40 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
|  | ↕ | ↕ | ↕ | ↕ | ↕ | ↕ | ↕ | ↕ | ↕ | ↕ | ↕ | ↕ | ↕ | ↕ | ↕ | ↕ |
| PLC : M001 | F | E | D | C | B | A | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |

When press F1 key of PMU (bit 2 of buffer 40), for example, the bit 2 of M001 word (M0012) of PMU will be turned on.

Now, save the setting as a file. (*.lnk) The following figure shows the ‘Save as’ message window.

- The screen of PMU after all setting is completed.

bit 0

of buffer 40

bit 1

of buffer 40

bit 2

of buffer 40

bit 3

of buffer 40

bit 0

of buffer 41

bit 1

of buffer 41