1. On/Off of a bit ( TOUCH TAG, FUNCTION KEY TAG, LAMP TAG )

  - Introduction

By using the Touch / Function key of PMU, turns on / off the contacts of PLC and indicates the status on the screen of PMU.

2

Example)

3

4

1

Turn on M0010 bit of K200S by touch the touch tag 1.

Turn on M0011 bit of K200S by touch the touch tag 2.

Turn on M0012 bit of K200S by pushing the F1 function key at the right side of PMU.

Turn on M0013 bit of K200S by pushing the F2 function key at the right side of PMU.

The lamp 3 will turn on when the M0020 bit of PLC K200S is turned on.

The lamp 4 will turn on when the M0021 bit of PLC K200S is turned on.

- System configuration

Built-in Cnet (4,5,7pin)

PMO-300S (RS232C option)

K200S CPU        PMO-300S

4                   2

7                   3

5                   5

1-2 Edit the screen of PMU

Run the Project manager software.

- Create a main screen

When you install PMU-MASTER software, 6 kinds of file managers will be created in Program group of Start menu.

- Select Project Manager in the PMU-MASTER Program group.

- Select PMU type and click OK button.

- If the ‘Work type’ windows appears, press the ‘Cancel’ button and choose ‘File – Change PMU TYPE’, and then select the PMU type.

- Select Editor-Screen Editor in the full down menu to create a drawing file.

- Select Project Manager in the PMU-MASTER Program group.

- Select PMU type and click OK button.