- Make a ‘Lamp’ tag

Select Tag Insert - Lamp Tag and move cursor into the circle and click.

Tag

The following message window appears, and input the name and buffer

and bit for the lamp tag.

Non-duplicated name

The lamp will turn on (white) when the bit 0 of buffer 41 is on, and turn off (black) when the bit 0 is off.

Register another lamp tag as same way.

- Saving files and test run (simulation)

Saving files

Give a name of file as numeric character (1 ~ 999)

- Simulation

Choose Others – Simulation in menu, and click ‘OK’ button in Screen Select window.

Input 40 at buffer item in ‘Enter Tool’ window, and monitor the changes of buffer 40 with pressing touch button and function keys.

Input the ‘41’ at buffer, and ‘1’ at value. It means turn on the bit 0 of buffer 41.

Monitor the changes of lamp tag with different values (2, 3, etc)