1-5 Transfer project file to PMU

- Make a project file

Run the Project manager software, and press the ‘Work Type’ button.

Select ‘make project file’ in the ‘Work Type’ window.

Choose a Work Directory at which the project file will be stored.

Choose all source files (.scr, .sub, .lnk, etc.) by double-click or ‘Select’ button.

After all source files are selected, press the ‘Make PRJ’ button.

Type the name of project file, and press ‘OK’ button.

1-6 Setup the PMU300

- Power on the PMU300.

- Press ‘F2 Initial Setup’ after the following menu is displayed.

F1  Diagnosis

F2  Initial Setup

F3  Transfer

F4 Simulation

F5 Run

PMU 300  MAIN MENU

- Press ‘F1 RUN MODE SETUP’ after the following screen appeared.

F1 Run mode setup

F2 Serial Setup

F4 Date/Time Setup

Main Menu/Initial Mode Setup

F3 System Setup

F5 Memory Initialization

F6 Link Information

- Run mode setup

This mode set operation method, initial screen number, initial mode and used memory in operation.

Set Initial Operating Screen to ‘number1’ (1.scr)

Set the Power ON Mode to ‘Menu’

Press ‘Save’ button after setup is finished.

Initial Mode Setup/Run Setup

Initial Operating Screen

Power ON Mode

0001

Menu

Operation

Save

\

\

\

\

- Operating Method

- Select Operation setup in the initial mode setup or press F1 function key.

- To select the left menu (Initial Operating Screen, Power ON Mode, etc.)

- Press  “↑ “ key or press touch key to be edited

Press   “↓ ” key or press touch key to be edited

- ‘Initial screen number (1-999)’ means the first screen to show up in the operation mode.

A. Enter with Touch key: After touch the tag, enter the screen number for initial screen.

Then presses enter key.

Cancel : Cancel entering data

Delete : Deletes entered all numbers

← : Deletes last entered number

B. Enter with Function Key : Presses enter key after enter the screen number in the right side of the screen.

- Power ON mode

When the power of main unit gets on, a user can select direct operation or main menu.

To select the menu, use F0 to F9 keys

- Press ‘Serial Setup’ button after the screen is changed to ‘Initial Mode Setup’ screen. Press ‘Save’ button after setup is finished.

Baud rate (300-38400)

Data bit/Stop bit

Parity bit

Interface

Station number (0-31)

Initial Mode Setup / Serial Setup

Save

[19200]

7/1   7/2   8/1   8/2

None	   Even 	   Odd

RS-232C        RS-422

00

- Setup for transmission preparation

Press ‘Transfer’ button in the main menu, and ‘ENT’ button. At this time, ignore the password.

Designate password and press enter key to execute !

Password : [0000]

< PC  Main Unit >

ENT(Execution)

Main Menu / Transfer

Baud rate: 19200 Baud rate

Data Length:  8 Bit

Stop Bit:  1 Bit

Parity Bit:  None

Interface:   RS-232C

- Transfer the ‘PMU300.PRJ’ file to PMU.

PC Serial Port(9p)     PMU Serial Port(PMO-300S,9p)

2                       2

3                       3

5

(Cable connection for project file transmission)

1) Setup the COM port

Assign the COM port to be used for transfer.

- Press the ‘Send’ button at the bottom of project manager, and press ‘Start’ button after the communication window is displayed.

The screen when the transmission is completed normally

1-7 Operating PMU

After the transmission is completed, press any key to escape from ‘Transfer Mode’.

Press ‘F5 Run Key’ in the following main menu to enter ‘Run Mode’.

F1  Diagnosis

F2  Initial Setup

F3  Transfer

F4 Simulation

F5 Run

PMU 300  MAIN MENU

- PLC program

The following example is a simple program for check the communication status between PLC and PMU.

Input the following sequence program into the PLC, and then run the PLC.

Monitor the M0010 ~ M0013 with pressing touch tag and function key of PMU. Also, monitoring the lamp tag with change the P0000 and P0001 of PLC.