Chapter 2. SPECIFICATION

This chapter explains the general specifications, performance specifications and functions.

# 2.1 General Specifications

| No | Item | Specifications |  |  |  |  |  |  | Standards |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Operating temperature | 0 ~ 55 C(32~131℉) |  |  |  |  |  |  |  |
| 2 | Storage temperature | -25 ~ +70 C(-13~158℉) |  |  |  |  |  |  |  |
| 3 | Operating humidity | 5 ~ 95%RH, non-condensing. |  |  |  |  |  |  |  |
| 4 | Storage humidity | 5 ~ 95%RH, non-condensing. |  |  |  |  |  |  |  |
| 5 | Vibration | Occasional vibration |  |  |  |  |  |  | IEC 1131-2 |
|  |  | Frequency |  | Acceleration |  | Amplitude |  | Sweep Count |  |
|  |  | 10 f57 Hz |  | - |  | 0.075 mm |  | 10 times per axis, on X,Y, Z axis |  |
|  |  | 57f150 Hz |  | 9.8 m/s 2{1 G} |  | - |  |  |  |
|  |  | Continuous vibration |  |  |  |  |  |  |  |
|  |  | Frequency |  | Acceleration |  | Amplitude |  |  |  |
|  |  | 10f <57 Hz |  | - |  | 0.035 mm |  |  |  |
|  |  | 57f150 Hz |  | 4.9 m/s2 {0.5G} |  | - |  |  |  |
| 6 | Shocks | - Maximum shock acceleration: 147 m/s2{15G} - Duration time: 11 ms - Pulse wave: half sine pulse (3 shocks per axis, on X,Y,Z axis) |  |  |  |  |  |  | IEC 1131-2 |
| 7 | Noise Immunity | Impulse Noise |  1,500 V |  |  |  |  |  |  |
|  |  | Electrostatic Discharge | Voltage : 4 kV (contact discharge) |  |  |  |  |  | IEC 1131-2, IEC 801-2 |
|  |  | Radiated Susceptibility | 27 ~ 500 MHz, 10 V/m |  |  |  |  |  | IEC 1131-2, IEC 801-3 |
|  |  | Fast transient / burst noise | Item |  | Power supply |  | Digital I/O (>24V) | Digital I/O (<24V) Analog I/O interface | IEC 1131-2, IEC 801-4 |
|  |  |  | Voltage |  | 2 kV |  | 1 kV | 0.25 kV |  |
| 8 | Operating ambience | Free from corrosive gases and excessive dust |  |  |  |  |  |  |  |
| 9 | Altitude | 2,000 m(6,562ft) or less |  |  |  |  |  |  |  |
| 10 | Pollution | 2 or less |  |  |  |  |  |  |  |
| 11 | Cooling method | Self-cooling |  |  |  |  |  |  |  |

| ## HINT |  |
| --- | --- |
| - IEC (International Electromechanical Commission) : An international civilian institute who creates and determines international standards in area of electric’s and electronics. - Pollution :An indicator, which indicates degrees of pollution that determines insulation performance of equipment. Pollution 2 means that nonconductive pollution usually occurs but that temporal conduction occurs with condensation. |  |

# 2.2 Performance Specifications

[ G6F-POPA ]

| Item |  | Specifications |
| --- | --- | --- |
|  |  | G6F-POPA |
| Output type |  | Pulse output (A/B type) |
| Number of control axis |  | Two ( simultaneous or independent) |
| Interpolation |  | Linear interpolations |
| Positioning data | Capacity | 300 points per axis (The operating data of step numbers are start from “0” to “299”) |
|  | Setting method | Input from S/W package |
| Position -ing | Method | Absolute / incremental method can be selected for each axis |
|  | Positioning command | - 16,744,447 to + 16,744,447 ( Unit : pulse) |
|  | Speed | 10 to 200,000 (pulse/sec) ( Speed data for X and Y axis are 128 ) |
|  | Acceleration /deceleration | Trapezoidal |
|  | Backlash compensation | 0 to 999 pulses |
|  | Zero point compensation |  999 pulses |
|  | Operation method | Single positioning |
|  |  | Repetitive positioning |
|  |  | Automatic positioning |
|  |  | Continuous positioning |
|  |  | Constant access positioning |
|  |  | Constant access positioning with the determined positions |
| Homing |  | Reducing time differs by toggles of the zero point L/S. There exist 3 methods by the time that the upper/lower limit or zero point L/S is reached. |
| M code function |  | 256 data (After/With mode) |
| Zone setting function |  | 3 different zones can be set. |
| Manual operation function |  | Jog operation, Manual pulse operation, Inching and Teaching. |
| Floating zero point set function |  | Setting the zero point by S/W at the point that the user wants to set. |
| Present value Preset |  | This function changes the present position to an arbitrary position. |