7.3 SAMPLE PROGRAM(MASTER-K)

7.3.1 Training kit system configuration

1.Positioning module is mounted at ”slot 2” on the basic base

2.The movement length of positioning kit is 0 ~ 400mm

3.The pitch of ball screw is 5mm

4.The distance per a rotation of shaft is 5mm

5.Feed back pulse form encoder to servo driver is 4000 per a rotation

6. Electric gear ratio is 1: 1.25(CMX/CDV)

7. It has not deceleration gear

8. The moving distance per a pulses 1m

The moving distance per a pulse

= ((The distance per a rotation of shaft)/( The number of pulse for a rotation of the shaft)) *

(Electric gear ratio) *(deceleration gear ratio)

= ( ( 5mm) / (4000) )* ( 1 / 1.25 ) * ( 1 ) = 1m

##### Program 1(Single)

1. Condition

- Push the input switch (P0000) then the pointer of the training kit returns to the origin point and stop

At that point ( Operation velocity :  high speed – 20Kpps / low speed – 3kpps )

- Push the input switch (P0001) then the pointer of the training kit moves to 150mm and stop at that point(Operation velocity : 40kpps )

- Push the input switch(P0001) again then the pointer of the training kit moves to 300mm and stop

at that point( Operation velocity : 20kpps )

- Push the input switch(P0001) once more then the pointer of training kit moves to 400mm and stop

at that point( Operation velocity : 15kpps )

- The input switch(P0002) is use for “error reset”

- The input switch(P0003) is use for “ clear the pulse out inhibit”

2. Operation time chart

Origin point

( 0mm)

First stop

(150mm)

Second stop

( 300mm)

Third stop

( 400mm)

Velocity

(40kpps)

Velocity

(20kpps)

Velocity

(15kpps)

3. Solution

1) Position parameter

(1) Basic parameter

(2) Data parameter

(3) Speed parameter

2) Position program

(1) Program

#####  Program 2(Auto)

1. Condition

(1) Push the input switch (P0000) then the pointer of the training kit returns to the origin point and stop

at that point ( Operation velocity : high speed – 20kpps / low speed – 3kpps )

(2) Push the input switch (P0001) then the pointer of the training kit moves to 150mm and stop at that

point( Operation velocity : 40kpps )

(3) After 3sec since the point was stopped at first point it restarts automatically to the second stop point and

stop at 300mm ( Operation velocity : 20kpps )

(4) After 3sec since the point was stopped at second point it restarts automatically to the third stop point

and stop at 400mm ( Operation velocity : 15kpps )

(5) The input switch(P0002) is use for “error reset”

(6) The input switch(P0003) is use for “ clear the pulse out inhibit”

2. Solution

All the position parameter and program same to solution of program 1 except for position data parameter

1) Data parameter

#####  Program 3 (Repeat)

1. Condition

- Push the input switch (P0000) then the pointer of the training kit returns to the origin point and

stop ( Operation velocity : high speed – 20kpps / low speed – 3kpps )

(2) Push the input switch (P0001) then the pointer of the training kit moves to 150mm(Operation

velocity : 40kpps ) and stop at that point for 1sec and restarts to the second position( Target

address : 300mm / Operation velocity : 20kpps ) and stop at that option for 1sec then returns to

the start option(Target address : 0mm / Operation velocity : 15kpps )

2. Operation time chart

Repeat

Repeat

Repeat

Velocity

(40kpps)

Velocity

(20kpps)

Velocity

(15kpps)

Origin point

( 0mm)

First stop

(150mm)

Second stop

( 300mm)

3. Solution

All the position parameter and program same to solution of program 1 except for position data parameter

1) Data parameter

#####  Program 4 (Position teaching / Velocity teaching)

1. Condition

(1) Push the input switch (P0000) then the pointer of the training kit returns to the origin point and

stop ( Operation velocity : high speed – 20kpps  /  low speed – 3kpps )

(2) Teaching the target address and operating velocity with data number by using PMU

 Input the first target address, operating velocity, data number and press set tag

 Input the second target address, operating velocity, data number and press set tag

 Input the third target address, operating velocity, data number and press set tag

(3) Push the input switch (P0001) then the pointer move to the first point as to defined on the PMU

(4) Push the input switch (P0001) again then the pointer move to the second point as to defined on the

PMU

(5) Push the input switch (P0001) once more then the pointer move to the third point as to defined

on the PMU

2. Operation time chart

Origin point

( 0mm)

First stop

Second stop

Third stop

First data

Second data

Third data

3. Solution

The basic position parameter is same to solution of program 1

1) Data parameter

3) Program

##### Device and Number interfaced with PMU

 D000~D001 : Position data to change target positioning point

 D0100 : Position data step number to change target positioning point

 D0110 : Velocity data Number to change target positioning speed

 D0111 : Velocity data  to change target positioning speed

 M0100 : A bit device to change target positioning point

 M0101 : A bit device to change target positioning speed