# Chapter 9. Troubleshooting

At this chapter, description and actions taken for various errors occurring with positioning module are explained. Occurring errors are largely divided into the non-critical and the critical.

Error occurrence is indicated by LED on the top of product and an error type may be checked with Teaching unit (G0F-PTUA) and software package (Position.exe).

## 9.1 Error type and correction

[Positioning module error code table]

| Error code | Description |
| --- | --- |
| 0 | Normal operation |
| 1 | Base positioning number exceeding setting range. |
| 2 | H/W error of base |
| 3 | Slot positioning number exceeding setting range. |
| 4 | Slot not mounted |
| 5 | Module name different |
| 6 | Axis range exceeding setting range. |
| 7 | CPU module gives out a control during Stop condition. |
| 8 | Common RAM error |
| 9 | FB control can not be executed since module is in operation/stop condition. |
| 10 | A function block of a new control is REQ before a previous control is completed. |
| 11 | Setting value of auxiliary input exceeding the range. |
| 13 | Stop-related controls or emergency stop is entered during the execution of function block. |
| 14 | NM or OR control is entered when not in normal speed phase of automatic operation. |
| 15 | A control issued when module zeroing is not complete. (Teaching) |
| 17 | H/W error |
| 18 | Watchdog time error |
| 19 | Interface with CPU error |
| 20 | When external emergency stop is entered |
| 21 | When internal emergency stop is entered |
| 22 | When external H/W lower limit switch is input |
| 23 | When external H/W upper limit switch is input |
| 24 | Current position exceeding lower boundary of S/W address range. |
| 25 | Current position exceeding upper boundary of S/W address range. |
| 26 | A number other than original operation mode number is entered. |
| 27 | Positioning data address exceeding the allowed range. |
| 28 | Dwell time exceeding the allowed range. (0~999) |
| 36 | The operation following automatic or continuous operation is incorrect. |
| 37 | Interpolation operation is selected during continuous, constant speed or positioning constant speed operation. |

| Error code | Description |
| --- | --- |
| 48 | Deceleration-related control issued during JOG or RTP control execution |
| 49 | Deceleration-related control issued during Dwell zeroing. |
| 56 | NM, OR or speed change control issued when not in constant speed phase by operation control. |
| 57 | NM or OR control issued in constant speed or positioning constant speed operation. Speed change control issued in continuous, constant speed or positioning constant speed operation |
| 58 | NM control issued in modes other than continuous operation mode. |
| 59 | N-M control issued when not in operation. |
| 60 | OR control issued but OR is set invalid. |
| 61 | Rotation direction can be changed with N_M control. |
| 66 | Out of positioning data to proceed with Data number exceeding the range when position teaching or speed teaching control issued |
| 67 | OR value in percentage exceeding allowed range 1~55(x 10%). |
| 68 | Inposition value exceeding allowed range (-999 ~ +999). |
|  | Inching value exceeding allowed range (1 ~ 99). |
| 69 | Position address set with preset control exceeding upper or lower limits. |
| 76 | Operation control issued when zero point is not decided and data type is absolute. |
| 77 | Position teaching control is entered when zero point is not decided. |
| 86 | Speed exceeding the range. (faster than the maximum speed, slower than the bias speed, or set as “0) |
| 87 | Lower speed of zeroing set faster than the higher speed or slower than the bias speed |
| 88 | Short cut speed calculated with line interpolation exceeding allowed range |
| 89 | Constant speed or deceleration time exceeding allowed range (Max. 65,535 ms) |
| 96 | A control issued when output is prohibited |
| 97 | A control issued during busy |
| 98 | Zeroing and start interpolation controls are issued when CPU module is stopped. |
| 106 | Parameter data are incorrect. |
| 107 | Data manipulation result exceeding the range |
| 116 | Exceeding variation counter range |
| 126 | Synchronous operation mode error |
| 127 | Station number setting error |
| 136 | Slave-axis failure |
| 137 | 485 communication error |
| 138 | Incorrect configuration of synchronous axis |
| 146 | Radius input exceeding the range in circular arc interpolation operation |

## 9.2 Trouble diagnosis

### 9.2.1 Positioning operation error

Manual

Operation?

Positioning not executed.

Yes

Check PLC CPU mode.

RUN

Check pulse output mode.

Corresponds to drive unit?

Are positioning module and drive correctly wired?

Is external output normal?

H/W failure

Contact service center.

E N D

Set to run mode

Adjust pulse output mode(type A,B) of parameter as for drive unit

Re-wire positioning module and drive unit.

Drive unit failure

No

No

No

Yes

No

Yes

Yes

Yes

### 9.2.2 Positioning speed is different from the setting speed

Set

Correctly?

Positioning setting speed is different from the actual speed.

No

Check parameter, positioning data and speed data setting.

Setting speed > Speed limit

Interpolation

Operation?

Has PLC scents speed change command?

Check external output.

Same with the setting value?

E N D

Enter correct data value.

Set setting speed slower than speed limit.

Speed changes depending on moving distance(not error)

Speed change by override, next move or speed change common

H/W failure

Contact Service center.

Yes

Yes

Yes

No

Yes

No

No

No

Yes

### 9.2.3 Abnormal stop during positioning

Abnormal stoppage during positioning

Check if pause-related command exists in PLC program.

Pause program exists.

Yes

Remove the program and start again.

Check if noise sources like

Is there a noise source?

Operate after pausing the noise source.

Stop?

Establish measures for noise

E N D

Stop?

Drive failure

Contact the service center

No

No

Yes

Yes

Yes

No

No

Zeroing error

9.2.4 Zeroing error

Monitor zeroing status with a software package

Is zero point limit switch on?

Zeroing speed changed to low?

Z-phase signal on?

Z-phase signal output from servo driver?

E N D

Servo drive failure

H/W failure

Contact the service center

Check the wiring of zero point limit switch

Yes

No

No

No

No

Yes

Yes

Yes