1. Features

- GLOFA Etherent system supports International Standard Specification. (IEEE 802.3)

- GLOFA Etherent system supports International Standard Protocol. (TCP, IP, UDP, ARP)

- GLOFA Etherent system supports International Standard Cable Specification.

- 10 Base 5

- 10 Base 2

- 10 Base T

- Various Communication Service

- Dedicated Protocol Service (Port 2004)

- Function Block Service (User Defined)

- High Speed Link Service (Port 2006)

- GMWin Service (Port 2002)

- The Maximum Number of Connection is 16. (Dedicated Protocol Service + FB Service)

- By using Frame Editor, Easy Programming for Communication with other equipment is

possible.

- Max. 4 Modules can be installed in main base

# Caution

Ethernet Module requires . . . . .

 GLOFA PLC GMWIN Program Tool : Ver 3.0 or higher

 GLOFA GM1/2 CPU : Ver 1.2 or higher

 GLOFA GM3 CPU : Ver 1.8 or higher

2. Specification

2-1 Performance Specification

|  |  | Specification |  |  |
| --- | --- | --- | --- | --- |
|  |  | 10 Base 5 | 10 Base 2 | 10 Base T |
| Trans. Spec. | Transmission Speed | 10Mbps |  |  |
|  | Transmission Method | Baseband |  |  |
|  | Max. Distance | 2500m | 925m | - |
|  | Max. Distance (Segment) | 500m | 185m | 100m (Node↔HUB) |
|  | Max. Nodes | 100/Segment | 30/Segment | Dependant to HHB |
|  | Min. Distance Between Nodes | 2.5m | 0.5m | - |
|  | Max Protocol Size | 1500 Byte |  |  |
|  | Access Method | CSMA/CD |  |  |
|  | Frame Error Check | CRC 16 = X15 + X14 + X13+ .... + X2 + X + 1 |  |  |
| Basic Spec. | DC 5V, Internal Consumption Current | 530Ma |  |  |
|  | Dimension | 250H x 35W x 140.7D |  |  |
|  | Mass | 460g |  |  |

2-2 Names of Parts

- 1) Front Part

LED Diaplay

10 Base T Connector

10 Base 5 Connector

External Power

Supply Terminal

& Display

(DC.12V,

In case of

10 Base 5 only)

Mode Select Switch

0: Run (On Line)

1: Test Mode 1

2: Test Mode 2

| No | Display | Description |
| --- | --- | --- |
| 0 | RUN | Power & Module Initializing is normal |
| 1 | CPU I/F RUN | Exchange(CPU↔Module) is Normal |
| 2 | FB -SERVICE | Turns on during execution FB Service |
| 3 | HS-SERVICE | Turns on during execution High Speed Link Service |
| 4 | GMWIN - SERVICE | Turns on during execution GMWin Service |
| 5 | GLOFA - SERVICE | Turns on during execution dedicated Protocol Service |
| 6 | FTP- SERVICE | Turns on during execution FTP Service (Not support yet) |
| 7 | H/W -ERROR | Occurring Fatal Hardware Error |
| 8 | -- | Not Used |
| 9 | -- | Not Used |
| 10 | 10BASE5 Enable | 10 Base 5 Enable & Ready |
| 11 | 10BASE2 Enable | 10 Base 2 Enable & Ready |
| 12 | 10BASE-T LINK | 10 Base T Connected |
| 13 | 10BASE-T PLRTY | Pority 10 Base T |
| 14 | TX | Turns on during Transmitting |
| 15 | RX | Turns on during receiving |

10 Base 2 Connector

3. System Configuration

3-1. 10 Base 5

Transceiver

Transceiver

Terminator

■                                                                                   ■

Transceiver Cable (Max. 50m)

Segment (Max. 500m)

3-2. 10 Base 2

T-Type Connector (BNC Connector & Terminator)

BNC Cable

Segment (Max. 185m)

3-3. 10 Base T

HUB

UTP/STP Cable

UTP/STP Cable

Max. 100m

4. Communication Service

4-1. Dedicated Protocol Service

1. Concept

1) This service make other nodes enable reading / writing information and data memory

in PLC CPU directly by using dedicated protocols built in PLC Ethernet module.

(G3L-EUEA / G4L-EUEA).

2) CPU control function (stop/pause/run) and PLC Program up/down loading is also possible

by this service.

3) TCP Port 2004 is assigned for this service and user cannot change this port no.

4) This service is mainly used for communicating between GLOFA PLC and computer.

2. Operation Flow

GM4 CPU

G4L-EUEA

①Connection Request

Port

2004

⑤Search &

Exchange

②Connection complete

Data

Memory

Internal

Memory

③Command

COMM

⑥Response

④

Application

User Program

(Visual C++,

MMI Package)

Communication

Processor

- ~ ② TCP Connection is established by other node’s(computer) connection request signal

to TCP port 2004.

- Other node sends the command frame created by user application software to Internal memory of PLC Ethernet module via TCP port 2004.

- Communication Processor of Ethernet module analyze and checks command frame.

- Datas requested by other node are exchanged between Ethernet module and PLC CPU.

- Ethernet module sends the response frame to other node.

4-2. Function Block (FB) Service

1. Concept

- Function Block service is used when the other node has a different protocol form from

GOLFA ethernet module and user intend to communicate with other node by using

other node’s protocol.

- Data frame can be easily created by frame editor tool and that frame file can be used

with Function Block in user’s PLC program.

3) This service is mainly used for communicating between GLOFA PLC and other ethernet

device which has an opened protocol format.

2. Operation Flow

GM4 CPU

G4L-EUEA

①Connection Request

②Connection complete

Internal

Memory

- Connection

Request

④Command

Function Block

E_CONN

COMM

③Command

⑤Response

Function Block

TCP_SEND

⑥Response

COMM

Application

User Program

Function Block

TCP_RCV

- ~ ② TCP Connection is established by execution of ‘E_CONN’ function block in PLC user

program.

③ By using ‘TCP_SEND’ function block, PLC CPU sends the command frame created by

frame editor tool to Ethernet module.

④ Ethernet module sends the command frame to other node via TCP port (Auto assigned)

⑤ Other node sends the response frame requested to Ethernet module.

- By using ‘TCP_RCV’ fuction block, PLC CPU receives the response frame from

Ethernet module

4-3. High Speed Link Service

- Concept

- High speed link service transmits data specified by link parameter to other nodes

cyclically.

- By just setting link parameter, All nodes can share data memory simply

- TCP Port 2006 is assigned for this service and user cannot change this port no.

- This service is mainly used for communicating between GLOFA PLCs.

- This service has the same concept as High speed link of F-NET.

4. Operation Procedure

Ethernet Parameter Setting

1.Common Basic

Setting

- IP Address

- Subnet mask

- Gateway .etc

FB Service

Dedicated Service

HS Link Service

2. Communication Frame

Creation by using

Frame Editor

Down Loading Ethernet Parameter

2. High Speed Link

Parameter setting

by using GMWin

3. Making the program

by using Function Block

& Frame file

E_CONN

TCP_SEND

Down Loading

Ladder Program & Parameter

5. Example (Dedicated Protocol Service)

5-1. Example Overview

By using GLOFA’s dedicated protocol, IBM PC reads data memory (MW100)

In the PLC CPU and displays results on the monitor screen.

5-2. System Configuration

IBM Compatible

Computer

GM4 CPU / G4L-EUEA

Command

HUB

Response

UTP Cable

5-3. Test Environment

|  | Computer | GLOFA |
| --- | --- | --- |
| CPU | Pentium 133 | GM4 CPU |
| Ethernet Module | 3C509 (3COM) | G4L-EUEA |
| Cable | 10 Base T | 10 Base T |
| O/S | Windows95 | - |
| Development Language | MS Visual Basic | - |
| Protocol | TCP/IP (Microsoft) | TCP/IP |
| IP Address | 165.243.93.1 | 165.243.93.2 |
| Port No | 2000 | 2004 (Fixed) |
| Client / Server | Client | Server |
| Communication Service | GLOFA Dedicated Protocol Service |  |

5-4. Setting Ethernet Basic Parameter

- Ethernet frame editor tool is consist of two part, Basic parameter and Frame list.

- In part of basic parameter, the parameter related Ethernet is decided

- In case of dedicated protocol service, frame list is not required to be set.

- IP Address : Sets IP Address of Ethernet module(G3L-EUEA/G4L-EUEA)

- Subnet Mask : This parameter means whether GLOFA (EUEA) and other node

are placed in the same network or not.

(255.255.255.0 means two node are placed in the same network)

- Gateway : In case that GLOFA (EUEA) and other node communicate via gateway,

sets IP Address of that gateway.

- HS Station No. : Sets High speed link station no. (available only in case of HS service)

- Connection No (Channel No) : Sets the number of TCP connection will be used for

Dedicated service.

Ethernet Module (IP Address)

Connection established

Port No

Connection No

- Connection Waiting Time out : If connection request signal is not received from other

node within the time to be set, an error occurs (available only in case of FB service)

- Disconnection Waiting Time out : If ACK signal to disconnection request of GLOFA

PLC Is not received from other node within the time to be set, GLOFA(EUEA) close

that connection.

- RX Waiting Time out : If command is not received form other node within the time to be

set, GLOFA(EUEA) close the connection. (available only in case of Dedicated Protocol

service)

- Retry Limit : Sets the number of retry

5-5. Computer Application Software (Sample software developed by visual basic)

IP Address of GM4 (G4L-EUEA)

Variable Address to be monitored

- Set the IP Address of GM4 and variable address to be monitored.

- Click ‘ Connect ‘ and then ‘ Read ’ or ‘ Read Cont ’

- Value of variable(%MW100), current response time and minimum/maximum response time

will be displayed.

6. Example (FB Service)

6-1. Example Overview

By using Function Block and frame editor tool, GLOFA GM4 reads data memory

of MELSEC A2U (D100 ~ D102, 3words)

6-2. System Configuration

GM4 CPU / G4L-EUEA

A2U CPU / AJ71E71-S3

Command

Response

HUB

UTP Cable

Response(0)

Response(1)

Response(2)

D100

D101

D102

6-3. Test Environment

|  | MELSEC | GLOFA |
| --- | --- | --- |
| CPU | A2U CPU | GM4 CPU |
| Ethernet Module | AJ71E71-S3 | G4L-EUEA |
| Cable | 10 Base T | 10 Base T |
| Protocol | TCP/IP | TCP/IP |
| IP Address | 165.243.93.3 | 165.243.93.2 |
| Port No | 3000 | 2004 (Fixed) |
| Client / Server | Server | Client |
| Communication Service | GLOFA FB Service |  |

6-4. Setting Ethernet Basic Parameter

- Basic Ethernet Parameter is the same as case of Dedicated Service.

6-5. Frame Editing (Other node’s protocol editing)

- AJ71E71-S3’s (MELSEC Ethernet module) Protocol frame (Command/Response format)

can be easily created by frame editor tool and that frame will be used with Function Block

in GLOFA PLC program.

- AJ71E71-S3’s command and response format is as below

- Command Frame Format (GM4→A2U, Hexadecimal code)

01

Read

Request

1 Byte

FF

Self

Station

1 Byte

0A

Waiting

Time

1 Byte

64000000

Head

Address(100)

4 Byte

00

Null

1 Byte

2044

Memory

Type(D)

2 Byte

03

Number

1 Byte

00

Null

1 Byte

D100 ~ D102

- Response Frame Format (A2U→GM4, Hexadecimal code)

81

Read

Response

1 Byte

00

Error

Code

1 Byte

1234

Data

2 Byte

5678

Data

2 Byte

90AB

Data

2 Byte

3) Execute frame editor and create AJ71E71-S3’s command / response frame.

Group name will be used as a element of

Connection method in function block (E_CONN)

Frame name will be used in function

block (TCP_SEND)

Send Frame Editing

01

Read

Request

FF

Self

Station

0A

Waiting

Time

64000000

Head

Address(100)

00

Null

2044

Memory

Type(D)

03

Number

00

Null

AJ71E71-S3’s

Command frame

♧Point !

Command / Response Frame can be devided as some segment by frame editor.

User need to optimize frame structre by this segmentation, Some parts as constant string,

the other parts as variable.

For example, two cases below has a same result.

Case 1 (All constant string)                  Case 2 (Constant string + Array Variable)

16# 2044

16# 0300

Array (0)

Array (1)

MOVE

In the unit

Of Byte

Frame name will be used in function

block (TCP_RCV)

Receive Frame Editing

6 Byte

81

Read

Response

#### 00

Error

Code

#### 1234

Data

(D100)

5678

Data

(D101)

9ABC

Data

(D102)

AJ71E71-S3’s

Response frame

6-6. PLC Programming (Function Block Programming)

- It is required to insert ‘Communication Library’ (COMMUNI.4FB) to GMWin

before function block programming.

- Function Block

| E_CONN | Description |
| --- | --- |
| ##### E_CONN EN NDR NET_NO ERR IP ADDR. STATUS SD PORT CH_EN METHOD CH_NO. BOOL USINT STRING UINT STRING USINT BOOL BOOL USINT BOOL | <Input> - EN : During on status, executes Continuously - NET No. : Slot no. module installed in. - IP ADDR. : IP Address of other node. (Active Conn.) - SD PORT : Port no. of other node. (Active Conn.) - METHOD *: Frame group name and Mode - CH_ NO. : Channel no. (0 ~ 15) = Connection no. |
|  | <Output> - NDR : Turns on when FB is executed normally. - ERR : Turns on when FB is executed abnormally - STATUS : Detail error code - CH_EN : Turns on when connection is established normally |

* METHOD : XXX_YYY

XXX = Group name configured by frame editor.

YYY = Connection (channel) mode (TCP_ACTIVE, TCP_PASSIVE, TCP_SELECT)

| TCP_SEND | Description |
| --- | --- |
| ##### TCP_SEND REQ NDR NET NO. ERR CH_NO. STATUS FRAME ARR_CNT DATA BOOL USINT USINT STRING UINT ARRAY BOOL BOOL USINT | <Input> - REQ : When input condition changes 0 to 1, (rising edge), executes 1 time - NET NO. : Slot no. module installed in. - CH_ NO. : Channel no. (0 ~ 15) = Connection no. - FRAME : Send frame name configured by frame editor - ARR_CNT : Number of array variable to be sent - DATA : Array variable |
|  | <Output> - NDR : Turns on when FB is executed normally. - ERR : Turns on when FB is executed abnormally - STATUS : Detail error code |

| TCP_RCV | Description |
| --- | --- |
| ##### TCP_RCV REQ NDR NET NO. ERR CH_NO. STATUS FRAME RCV_LEN DATA_LEN DATA BOOL USINT USINT STRING UINT ARRAY BOOL BOOL USINT | <Input> - REQ : When input condition changes 0 to 1, (rising edge), executes 1 time - NET NO. : Slot no. module installed in. - CH_ NO. : Channel no. (0 ~ 15) = Connection no. - FRAME : Receive frame name configured by frame editor - DATA_LEN : Number of array variable that FB receives - DATA : Array variable |
|  | <Output> - NDR : Turns on when FB is executed normally. - ERR : Turns on when FB is executed abnormally - STATUS : Detail error code - RCV_LEN : Number of array variable to be received |

3) Programming